MODEL “FL”

1-1/2” Deep with 1” Front Flange

EXTRUDED ALUMINUM BRICK VENTS

- Extruded alum. to be 6063 - T5 Alloy.  Minimum .125” thick.

- Blades are 1” deep, set at 45 angles, 1” on center.

- For any unit over 30” wide - do not mortar jamb sections.

- Screens: 18 X 14 aluminum mesh

| MODEL # | ORDER SIZE | FINISHED DIMENSIONS | MINIMUM OPENING REQUIRED | TIP TO TIP DIMENSIONS |
| --- | --- | --- | --- | --- |
| 825 | 8-1/8”W x 2-3/8”H | 8-3/8”W x 2-3/8”H | 8-1/2”W x 2-5/8”H | 10-1/8”W x 4-3/8”H |
| 847 | 8-1/8" x 4-3/4" | 8-3/8" x 4-3/4" | 8-1/2”W x 5”H | 10-1/8”W x 6-3/4”H |
| 808 | 8-1/8" x 7-3/4" | 8-3/8" x 7-3/4" | 8-1/2”W x 8”H | 10-1/8”W x 9-3/4”H |
| 123 | 12" x 2-3/8" | 12-1/4" x 2-3/8" | 12-1/2”W x 2-5/8”H | 14”W x 4-3/8”H |
| 125 | 12" x 4-3/4" | 12-1/4" x 4-3/4" | 12-1/2”W x 5”H | 14”W x 6-3/4”H |
| 128 | 12" x 7-3/4" | 12-1/4" x 7-3/4" | 12-1/2”W x 8”H | 14”W x 9-3/4”H |
| 1212 | 12" x 11-3/4" | 12-1/4" x 11-3/4" | 12-1/2”W x 12”H | 14”W x 13-3/4”H |
| 157 | 15-5/8" x 7-3/4" | 15-7/8" x 7-3/4" | 16”W x 8”H | 17-5/8”W x 9-3/4”H |
| 1516 | 15-5/8" x 15-3/4" | 15-7/8" x 15-3/4" | 16”W x 16”H | 17-5/8”W x 17-3/4”H |
| 162 | 16-1/2" x 2-3/8" | 16-3/4" x 2-3/8" | 17”W x 2-5/8”H | 18-1/2”W x 4-3/8”H |
| 164 | 16-1/2" x 4-3/4" | 16-3/4" x 4-3/4" | 17”W x 5”H | 18-1/2”W x 6-3/4”H |
| 168 | 16-1/2" x 7-3/4" | 16-3/4" x 7-3/4" | 17”W x 8”H | 18-1/2”W x 9-3/4”H |
| 1616 | 16-1/2" x 15-3/4" | 16-3/4" x 15-3/4" | 17”W x 16”H | 18-1/2”W x 17-3/4”H |
| 242 | 24" x 2-3/8" | 24-1/4" x 2-3/8" | 24-1/2”W x 2-5/8”H | 26”W x 4-3/8”H |
| 244 | 24" x 4-3/4" | 24-1/4" x 4-3/4" | 24-1/2”W x 5”H | 26”W x 6-3/4”H |
| 248 | 24" x 7-3/4" | 24-1/4" x 7-3/4" | 24-1/2”W x 8”H | 26”W x 9-3/4”H |
| 3208 | 32" x 7-3/4" | 32-1/4" x 7-3/4" | 32-1/2”W x 8”H | 34”W x 9-3/4”H |
| 4808 | 48" x 7-3/4" | 48-1/4" x 7-3/4" | 48-1/2”W x 8”H | 50”W x 9-3/4”H |

| # SUNVENT INDUSTRIES By SYLRO SALES CORP. ## 1 Industrial Drive, #26 Pelham NH, 03076 Toll Free: 1-800-325-4115 ### In NH: 603-595-4556 Fax: 603-595-4778 www.sunventindustries.com info@sunventindustries.com | PROJECT: |  |  |
| --- | --- | --- | --- |
|  | LOCATION: |  |  |
|  | ARCHITECT: |  |  |
|  | CONTRACTOR: |  | JOB NO: |
|  | AGENT: |  | ORDER NO: |
|  | DATE:7/30/2026 | REVISIONS: | DWG NO: FL |