TRG Pittsburg NH Log Yard

10 Amey Drive – right off Route 3 in Pittsburg, NH

TIMBER RESOURCE GROUP (TRG)

Log Buyers – John Rounds (603) 731-4032 & Craig Owen (603) 731-1903

Please contact Buyers for the latest prices, thanks.

SPRUCE AND FIR SPECIFICATIONS

Effective September 2018

Species Accepted	  Red, White, Black & Norway Spruce and Balsam Fir

Limbing		  All limbs to be cut flush with trunk.

Defects                            Broken stems & tops, butt shatter and splits, burned stems, forked tops, excessive crook &

sweep and dead trees are considered defects which do not meet specifications.

Pasture spruce	  Pasture spruce/fir not accepted.  These are logs with excessive knots, burls, swells and often

excessive taper as to produce unacceptable lumber.

Stems not meeting specifications will be reloaded onto delivering truck for return to the Seller.

Spruce/Fir Sawlogs

Log Lengths 		 	12'-14'-16' with 4" minimum trim; 6” maximum trim (especially on 16’)     16' is preferred.

Minimum Top Diameter     	5” inside bark

Rot:                        	No rot allowed

.

Scale Rule		    	Weight Scale – wood is paid by the ton

Spruce/Fir Tree Length High Grade

Minimum Butt Diameter:	7” inside the bark			Minimum Top Diameter:	4.5” inside the bark

Minimum Length:	Butt logs - 16 feet 4”             Whole Tree - 29 feet

Any butt logs slashed out of over-length stems must be included in the run to be weighed and

be produced into conventional log lengths of 16’ 4” only.

Butt Rot:                        7” to 9” butt diameter – no rot allowed;

10”+ larger butt diameter – up to 25% solid rot accepted on up to 5 stems per load.

Scale Rule		Weight scale – Wood is paid by the ton